Research paper Version R0.9.2

Using Advective Transport Phenomena In Modeling To Better Understand Mechanical Dispersion: Step One

Wat bedoel je precies met “active transport phenomena”? Bedoel je het meenemen van advection, of het analytisch modelleren van de advection (heteregeneous k-fields) als proces en dat meenemen in een model. Of alleen het analytisch beschrijven van dispersie zodanig dat dit overeenkomt met wat er in de praktijk wordt gemeten, zodat we dat kunnen begrijpen en voorspellen? Dus zonder model of door dit over een model heen te leggen?

Willem J. de Lange

Retired from: Unit Subsurface and Groundwater Systems, Deltares, Utrecht, the Netherlands

Corresponding author: Willem J. de Lange, Unit Subsurface and Groundwater Systems, Deltares, Utrecht, The Netherlands, P.O. Box 85467, 3508 AL, Utrecht, the Netherlands (wimlanglely@outlook.com)

Conflict of interest: None

Keywords: dispersion, advection, modeling, field-scale, plume-growth,

Article Impact Statement: The dispersive volume shift is presented as parameter to replace the classical dispersivity parameter in modeling.

Abstract

It is widely accepted hat dispersion of pollutants as process during their subsurface transport is essentially spatial velocity differences, rather than by the concentration gradients. The classical Fickian model, which assumes dispersion is driven by concentration gradients is still widely used (a few important references here), but that has been often proven insufficient to capture actual pollution plume spreading during subsurface groundwater driven transport (if possible one or two references here). The groundwater community has been wating for needs and eagerly awaits a theoretical framwork providing support for advection as the dominant driver (een theoretische ontwikkeling voor een theory kllinkt niet zo mooi: bijv. Has been waiting for a theory-based development which allows practical implementation and application) offering a reasonable and practical way to implement it while demonstrating its superior value in matchting model with reality as observed in numerous sites throughout the world.

In his introductory paper, “Advective Transport Phenomena”,  De Lange (2020), describes mechanical dispersion  as the spread of water particles generated by flow through heterogeneities at sub-model scale, thus making the ubiquitous above -microscale hetereogeneity of subsurface conductivity fields responsible for the observed phenomena. This papers presents (presents again? Dat had je toch al gedaan in voorgaande artikelen?) equations describing the growth of the spread of a plume or cloud of particles driven by groundwater flow as a function of traveled distance. The equations are founded on definitions of characteristic length (called spread), and characteristic thickness (called wake). Spread and wake are separately defined for the front and the tail of the plume and they have distinctly different values.

A new parameter is introduce, called the dispersive volume shift, which is derived from stochastic parameters that describe the aquifer’s heterogeneity and the distance traveled (in the time step?). This parameters differs from the well-known dispersivity , which is used in Fickian models to describe the solute dispersion to match field observations. This  dispersive volume shift should replace dispersivity as the main parameter in future numerical models.

As a demonstration, two actual pollution plumes are simulated starting shortly after their release. One of them was in a weakly heterogeneous aquifer and the other a strongly heterogeneous aquifer. To simulate, reported data for these two famous large field experiments were used.

Because the time step in this plume modeling is critical, future numerical implementations specification of the optimum time step and cell size to calculate the concentration from the dispersive mass-flux. Such specification requires robust testing?

Het lijkt geen goede zaak als je van tevoren uitvoerig onderzoek moet doen naar de grootte van de tijdstap en de celgrootte van je model voordat je deze methode zinvol kan toepassen. Ik denk dat er vanuit de theorie altijd een of ander criterium moet kunnen worden afgeleid, zoals een Peclet getal, waar de de tijdstap en of de celgrootte baseert. Je kan die niet baseren op gissen en missen. Dat is geen wetenschap.

Introduction

As one reviewer commented on a previous concept “The idea that mechanical dispersion is essentially driven by a velocity variations, rather than by the concentration gradients as in the classical Fickian model, is a widely accepted. Yet, it is a concept for which a practical numerical implementation is still lacking and unproven. The groundwater community has been waiting for a theory-based development that supports this theory and offers a reasonable and practical way to implement it.”

As a next step in the development of the method to properly model groundwater driven dispersion based on only Advective Transport Phenomena (ATP) De Lange (2020, 2022, 2024), the current paper aims to fill the theoretical gap pointed at above and to deepen its understanding. This paper introduces a new parameter to quantify dispersive mass transport, which would replace the classical dispersivity parameter in future numerical subsurface transport models.

De Lange 2020, describes the spread of water particles caused by conductivity variations in the aquifer at sub-model scale, i.e. at the “local scale” as understood by (Dagan 1989, figure 3.n) and not at “micro-scale” or pore-scale like it is assumed in classical dispersion theory (Bear, Molz, Neumann, Batu). The dimensions relevant for the dispersion process at the local scale are indicated in figure NN in Kinzelbach and Name (REF) (Neem ajb het Plaatje van Kinzelbach over, want niemand zal dat op willen zoeken, waardoor lezers in het ongwisse blijven over dit essentiële punt). The figure shows heterogeneous zones or gravel pockets in a sandy aquifer with extents in the range of one to tens of meters. The same extent range has been reported as characteristic for (all, some, many?)large field experiments, carried out throughout the world (De Lange, 2022).

Conceptually, ATP(“Adenosine triphosphate”, het belangrijkste molecuul in levende organismen) differ fundamentally from that classical Fickian type dispersion theory as explained by (Bear, Dagan); it leads to a set of equations that capture distinguished (“distinguished” is vaag, kan ook betekenen “zeer hoogstaand, geacht, weledelgestreng”) situations (Wat bedoel je nu dat je nieuwe vergelijkignen hebt. Dat die verschillende situaties (beter) beschrijven dan de oude. Zo ja welke dan ?). In ATP (Wat is ATP precies? Een zelfstandig naamwoord, een process, een mechanism, een theorie. Zoals ik het lees is het gewoon een meervoud. Maar een meervoud is geen ding. Ik zou iets doen als AT-theory (de theoriey gebaseerd op acvective transport of ATD (advective transport dispersion) of zoiets, dan is het een theorie), mechanical dispersion is onlcy cause by water particles moving with mutually different speeds along their flow paths, so only flow-path aligned velocity differences matter leaving transverse mixing out of the equation. In a transport model, it’s an additional processes superimposed on the average velocity in the aquifer, or in the cell of a model. In the present work, dispersion is considered to occur along a flow paths computed by a numerical model unable to handle local hetereogeneities, and is added to these flow paths. As a first step towards implementation in a numerical model, a set of applicable equations is presented, and a demonstration is given that demonstrates the model performance in capturing the development over time of two reported and widely cited pollution plumes that characterize a weakly and a strongly heterogeneous aquifer.

De Lange (2022), applied ATD to better model dispersion as observed in 14 large-scale field experiments carried out at nine well-documented sites in different parts of the world. The reported stochastic parameters describing the heterogeneity of those aquifers were used in the comparison between ATD and classical Fickian analysis (deze zin klopt niet met de voorgaande). The log conductivity variances and dispersivities reported for these 14 experiments span a range of over two orders of magnitude. Most of the dispersivities were derived from the variance of the spread of solvents divided by two times the length of the distance traveled by the plume’s center (Axness and Gelhar 1989, eq (3)). The average of these 146 ratios of reported and calculated dispersivities is 0.91,

Ik vermoed dat dit de voor de lezer de belangrijkte argument is om jouw theorie verder te bestuderen. Ik vermoed namelijk dat je bedoelt dat het gemiddelde van wat gepubliceerd is en de waarden die jij op baiss van jouw theorie berekent 91% is. Dus een bijna perfecte match. Als je dit wilt zeggen moet je dat duidelijk doen, anders wordt daar overheen gelezen.

while about 95 per cent of the calculated dispersivities remains within a factor 2 of the reported ones (mij is niet duideijk wie nu wat heeft gerapporteerd. Maak duidelijk onderscheid tussen wat uit de artikele kwam en wat jij hebt bepaald). De Lange (2022) concluded that the dispersivity will remain constant with growing travel distance for highly heterogeneous aquifers (dat wil zeggen de ratio van spread gedeeld door gemiddelde travel distance toch? Want de spread zelf groeit linear met de afstand als advecitie deominant is, toch?), while it will grow linearly with travel distance for homogeneous aquifers, while actual cases will be somewhere in between. Whereas applicability of the method of ATP in field situations was presented by De Lange(2022), the present work is a first step to actual numerical implementation.

The pictures in figure 1 are generated with the analytic element method (Strack, 1989), whereas those in figures 4 and 5 are generated in Microsoft365 Excel.

Methodology

Numerically modeling ATP (of Implemening ATD-theory in a numerical model)

A numerical groundwater flow and transport model usually computes a transient 3D distribution of flow, potential and concentration. Numerical models explicitly includes aquifer heterogeneity like layers and large known incursions at model- scale.

These numerical models also produce the traveled distance along a path-lines as well as consituent concentrations per computation cell. Both traveled distance and concentation at “model scale” are input to ATP. ATP results in a set of parameters for particular situations, each of which can replace dispersivity as used in classic groundwater mass transport modeling (REF waar staat REF voor???).

ATP descrives mechanical dispersion as the spreading of dissolved particles in a vertical cross-section that bends along the model-computed path line (maar het kan toch zijn dat pathlines op verschillend hoogten binnen de aquifer in verschillende richting lopen, zodat een vertical sectie eigenlijk niet mogelijk is?. The spreading is generated by the aquifer heterogeneity at “sub-model” or local-scale, i.e. within computation cells or in general, below the scale ignored by the numerical descretization. This approach presumes absence of incoming or outgoing flow perpendicular to the section, which is reasonable in regional groundwater flow as well as in radial flow to a well. (I don’t agree, a section towards the well also has no flow perpendicular to it, since it has the stream function as has the regular cros section).

Following (De Lange ,2020), the vertical section along the path-line is covered by stacked and repeated units called “domain A”, where A refers to “averaging”. Each domain A is regarded as a mixing box in which the particles are completely mixed, i.e. local dispersion is complete. The size of the domain A  is completely defined by the stochastic parameters that describe the heterogeneity of the aquifer at sub-model scale within the section. The theoretical basis for the approach with mixing cells of given size was given by (De Lange, 2020). The theory resulted into parameters that quantify the spread of the mixed particles per domain A. The equations led to relations between stochastic input and output parameters. The theory and application is is comparable to the relation (NO) presented by (Axness and Gelhar, 1983/9) between the stochastic parameters that characterize the aquifer heterogeneity and its longitudinal dispersivity. In each domain A, the spreading of water particles is assumed to grow linearly with traveled distance. This result from the use of stochastic paramters to describe the essentially unknown positions and sizes of the heterogeneous zones at the local scale within the aquifer .

Dit lijkt me niet juist. Juist omdat je ze niet individual kan beschrijven gebruik je stochastiek. Vandaar de herformlering.

How repetition of the dispersion process in subsequent domains A works out, was analysed and applied in De Lange (2022, 2024).

The physical process underlying the ATP

Figure 1 shows how the front and the tail of a pollution front distort due to an inclusion in a zone within the aquifer having a conductivity  that is remarkably different from that of the background. Two largely different situations are shown. At the top a large part of the incomimg water is channeled though a inclusion with a conductivity 5 times that of its background. The bottom figure shows the situation in which the incoming groundwater largely passes along an inclusion with a 5 times lower conductivity than its background. (Note that in the bottom figure, the number of streamlines near the inclusion was increased by a factor of 10 to better visualize the details of the flow around the inclusion.) The phenomena that these figure illustrate also define dispersion of a plume, which is different at the front and the tail. The upper picture shows what happens at the front. An initially vertical straight front of water particles (blue line at the left) attained a distorted shape (blue line at the right) after flowing through and along the highly conducive inclusion. The lower figure, on the other hand shows how the front is distorted when passing an inclusion with a low-conducive inclusion. In the top figure, the originally flat front at the left as attained a relatively large fast progressing bulb or front upon arriving at the right. Whereas the originally flat front at the left has gotten a tail upon arriving at the righ, after passing a low-conducive inclusion.

Figure 1. Vertical section with equipotential lines (red) and path-lines (green) for a specified travel time. See explanation in text. Blue lines indicate the initial and the final shape of the water particle front. (From: De Lange 2020)

In figure 1 top, the water particles inside the high-conductivity (high-k) zone travel faster than those outside in the aquifer. The faster traveling inside causes contraction of flow lines at the inflow and expansion of flow lines at the outflow boundary of the high-k zone. The height of the affected zone in the aquifer is called wake and is thicker than that of the high-k zone. The amount of water particles that can flow through the high-k zone is limited by the wake and is obviously to be smaller than thickness of the aquifer. This limits the velocity in the high-k zone as well as the forward growth of the plume front. (als de inclusive lang is kan je het gewoon uitrekenen k1D1 + k2D2 = kD, toch?)

In figure 1 at the bottom, the water particles passing thorugh the low-conductivity zone travel slower than those passing along it. The slower traveling causes expansion of flow lines at the inflow and contraction of flow lines at the outflow boundary of the low-k zone. The wake is thinner than the thickness of the low-k zone and shrinks zero when the conductivity inside the low-k zone becomes zero. (in which the length of the plume tail can become as long as the traveled distance. Nee, in de extreme situatie dat de doorkatendheid in de low-k zone nul is, verdwijnt de tail gewoon helemaal. Die wordt dan ook oneindig dun.). Maar goed high-k zones genereren foward progressing bulbs en low-k zones veroorzaken tails. Don’t worry about the extremes. Want in de extreme situatie gaat alle het water door de high-k zone en gaat er helemaal geen water door de low-k zone.

The size of the domains A

Following (De Lange 2020, 2022), the size of domain A is assessed from the following. The stochastic characteristic parameters λh, λv called the characteristic length in horizontal and vertical direction respectively and σ2lnk  called the log conductivity variance  (Axness and Gelhar, Dagan) are taken to characterize the aquifer heterogeneity at sub-model scale. The length and thickness of the zones in figure 1 are called Lh and Lv and are equal to twice the characteristic length λhand λv respectively (De Lange, 2020). The length LA and thickness DA of a domain A are related to Lh and Lv by the scale factor N according to  LA=LhN=2λhN and  DA=DhN=2λvN.  In (De lange, 2022), the value of N has been assessed from field observations to lie between 0.4 for homogenous aquifers and 0.3 for heterogeneous aquifers, which compare well to the results from the numerical analysis of the size of a boundary zone by (Jankovic et al, 2003). The dispersion process in domain A is completed if the traveled distance S equals the length of domain A, so for S=LA.

In a domain A, the spread ΔsA,j as occurs to the water particles depicted in figure 1 comes from the velocity difference inside and outside the conductivity zone integrated over the distance over which the velocity difference occurs, the so-called distortian distance Ldis,j  (figure 1), where j equals f for the front and t for the tail of the plume. Using Darcy’s law, the equation for the increase of the spread by domain number j becomes (De Lange, 2020):

ΔsA,j =  1 – κjηj Ldis,j					(1)

Where κj=ka/kc [-] is de conductivity contrast and ηj=na/nc [-] is the porosity ratio and index a indicates aquifer and index i indicates the conductivity zone (of de domain??). In the comparison with field experiments in (De Lange 2022), the spread was used as the standard deviation of the concentration distribution in the well-known equation (deze equation is echt niet well-known voor de zomaar geïnteresseerde lezer die hier dan natuurlijk gaat afhaken, want die kan je dan niet meer volgen) (Adams and Gelhar, 1992,(10)) that relates the dispersivity to the variance of the observed distribution divided by twice the traveled length. (geef gewoon de formule!) The equation for the affected thickness or the wake WA,j, named after (Eames and Bush, 1999), was derived from continuity of flow outside the wake in the undisturbed aquifer and in the center of the conductivity zone???  (De Lange, 2020):

WA,j=DA/(1+κj(1N-1)					(2)

From numerical simulations by (De Lange 2020), the distorted length appears to be about equal to the length of the conductivity zone plus the lengths of the contraction and expansion zones in the two wakes, so  Ldis,j=Lh+2WA,j. The conductivity contrast κj was derived from the logarithmic conductivity variance and differs for plume front and tail, according to 1κf= κt= eσ2lnk. These equations show that the ATP parameters are one to one related to the stochastic parameters that represent the aquifer heterogeneity at sub-model scale.

Consequences of conservation of mass

After flowing through a domain A, the average of the different travel distances of the water particles shown in figure 1 equals the distance, so S=LA. Continuity of flow causes the part of the water particles traveled faster than average to be equal to the part traveled slower, which are volumes shifted forward and backward relative to the average traveled distance. In figure 2, the shifted volumes are positioned at the boundary of a domain A which are named drift (forward) and reflux (backward) after Dagan and Lessof CHECK.

Figure 2. Vertical section. Scheme of spread and wake from figure 1, top to define drift and reflux for the shifted volumes across the boundary of a domain A.

Using rectangular shapes, the length of the volumes of the drift and reflux follow from continuity of flow (De Lange 2020) and are expressed by:

ΔsA,j,dr = ΔsA,j 1-βj 	 				(3)

ΔsA,j,ref =ΔsA,j βj						(4)

Where the wake fraction βj is defined by:

βj=WA,jDA						(5)

Dispersive change of mass

For a domain A , the dispersive change of mass occurs during period TA needed to pass length LA at the average velocity ua in the aquifer. The volumes VA,j of drift and reflux, the shaded zones in figure 2, are equal (mass balance) and described by:

VA,j= ΔsA,j,dr WA,j=ΔsA,j,ref1- WA,j			(6)

As mentioned in the introduction, the spreading in a domain A is assumed to grow linear with travel distance. Then, the specific shifted volume υj can be defined as  VA,j  per area of domain A and becomes, using (3) and (4):

υj=VA,jLADA= ΔsA,jLA1-βjβj  				(7)

The value of this parameter depends only on the stochastic parameters characterizing the aquifer heterogeneity. The shifted volume  ∆V∆T,j grows with the average traveled distance ∆S∆T, leading to:

∆V∆T,j=υj∆S∆T						(8)

Figure 3 shows the basic scheme of mechanical dispersion as follows from ATP to determine a description for the mass flux in modeling. Though figure 3 depicts a situation of a plume front (figure 1, top), the results of the following analysis are the same for a plume tail.

In addition to the transport by advection at average velocity (figure 3, bottom), mechanical dispersion generates shifts of equal volumes of drift and reflux across a cell boundary, comparable to those across the boundary of a domain A in figure 2. The grey scale in figure 3 indicates the concentration descending from left to right as interpolated in between the calculation nodes in the two cells. The volumes and shifts are represented by the rectangles and arrows respectively.

In figure 3, middle picture, the downstream cell (at right) “imports” mass from the upstream cell and vice versa “exports” mass by the reflux. The reverse occurs in the upstream cell. So, in addition to advection, mechanical dispersion causes gain of mass at the upstream boundary and loss of mass at the downstream boundary of a cell.

To determine the mass flux by the shifted volumes, figure 3, top shows the shifts in the concentration gradient as a result of the drift (black) and reflux (light grey).

Next, consider mechanical dispersion during an arbitrary time step ∆T in addition to the result of advection. In figure 3, the volumes of drift and reflux are shifted over a spread Δs∆T,j,k  where the index k points at the drift (dr) or the reflux (ref). The concentration is shifted upward and downward accordingly in figure 3, top. The gradient of the concentration does not change due to the shifts.

Figure 3. Vertical section. Impact of mechanical dispersion (middle) in addition to advective transport (bottom). The grey scale descending from left to right in the direction of flow represents the concentration interpolated in between calculation nodes (circles). After a time step ∆T, the volumes of drift and reflux are, relatively to the result of advection, shifted across the cell boundary causing import and export of mass in the downstream cell in addition to advection.

The downstream cell gains mass, from the shift of the drift in direction of the flow (positive) and the increase of the concentration (positive) leading to a positive mass flux (∆V∆T,jΔc∆T,j,dr). The downstream cell looses mass from the shift of the reflux in direction opposite to that of the flow (negative) and the decrease of the concentration (negative) leading to a positive mass flux (∆V∆T,jΔc∆T,j,ref). The total mass flux across the boundary becomes equal to  (∆V∆T,jc∆T,j,dr+Δc∆T,j,ref=∆V∆T,jc∆T,j). The concentration difference Δc∆T,j follows directly from the concentration gradient between two calculation nodes  ∆cnodes∆Xnodes and the spread ∆s∆T,j. Then, the shifted mass  ∆M∆T,j across the cell boundary in time step ∆T becomes:

∆M∆T,j=∆V∆T,jΔs∆T,j∆cnodes∆Xnodes				(9)

The spread Δs∆T,j over time step ∆T is derived from the spread in domain A in (1) using the  linear growth with the traveled distance ∆S∆T over the length of domain A LA:

Δs∆T,j=∆S∆TLAΔsA,j						(10)

For time step ∆T and using ua*∆T=∆S∆T, the dispersive mass flux in the aquifer becomes,

F∆T,j=∆M∆T,j∆T=ω∆T,j∆cnodes∆Xnodes ua				(11)

In which ω∆T,j is defined as the dispersive volume shift given by:

ω∆T,j=υj∆s∆T  						(12)

which differs from and replaces the description given in (De Lange 2020). Apart from the traveled distance ∆S∆T in (10) that comes from the numerical model, ω∆T,j is determined only by stochastic properties for the aquifer heterogeneity. The dispersive mass flux in (11) is comparable to the term describing mechanical dispersion in the classic PDE used in modeling (REF).

Equations for modeling

In the demonstration following, mechanical dispersion is computed in a Lagrangian framework (Bear 1979, p599), that is for a chain of cells moving with average aquifer velocity ua.

Figure 4  Vertical section. Mass exchange across the boundaries of a cell moving with average aquifer velocity.

Figure 4 shows the calculation scheme for a cell in a Lagrangian framework. For each boundary of a cell, the local concentration gradient follows from the concentrations at the calculation nodes. For the cell at the center, the mass exchange over both cell boundaries becomes:

∑∆M∆T,j=ω∆T,j[(ci-1- ci)∆xi-1,i+(ci+1- ci)∆xi,i+1]			(13)

The concentration change at the node in the cell after time step ∆T follows from averaging the mass change over the cell volume:

∆c,∆T,j=∑∆M,∆T,j∆Wcell						(14)

The demonstration concerns a plume traveling with average aquifer velocity. The cell moving with the plume center covers the remainder of the original injection and acts as a tail (j=t) at the upstream boundary and as a front (j=f) at the downstream boundary, for which the mass flux is composed of both front and tail, according to:

∑∆M,∆T,j=υA,t(ci-1- ci)∆xi,i-1+υA,f(ci+1- ci)∆xi+1,i∆S∆T		(15)

The mass flux in the equations (13) and (15) hold per unit area perpendicular to the flow direction.

Equation (13) shows similar to a first order FDM term (Bear and Verruijt, , eq.). However, (13) does not reflect an interpolation between nodal concentrations as explained above and the elaboration in equation (15) shows.

Demonstration: plume development at the sites Cape Cod and MADE.

To demonstrate the capabilities of the method in the present stage of development, a moving plume is simulated in the period just after the injection of a tracer in a weakly and a strongly heterogeneous aquifer for which input data comes from the sites at Cape Cod (Ref) and  MADE (REF).  The calculation is described in detail in appendix A, including the input data and the discretization. A user-friendly spreadsheet is provided.

In the first step, the mass change is calculated from the concentrations at the nodes in the previous step (13), (15). This step shows the closed mass balance throughout the computation. In the second step, the concentration change at the cell node is calculated from the mass change and cell size (14). The concentration in a cell changes if the sum of drifts of the front or tail covers the distance between the plume center and the cell center.

Table 1 Published values of stochastic parameters (from DL2)

| Nickname | Primary reference | σ2lnK | λv [m] | λh [m] | Da [m] |
| --- | --- | --- | --- | --- | --- |
| Cape Cod | (Garabedian et al. 1992) | 0.24 | 0.39 | 8 | 30 |
| MADE | (Adams and Gelhar 1992) | 7 | 1.5 | 12.3 | 10 |

The stochastic aquifer properties are site specific en given in Table 1 (from: DL2). The other input parameters are taken equal for both cases. The chosen average aquifer velocity of 0.5 [m/day] is close to the reported values in both cases. The injection is simulated by a concentration of 100 % in the center cells with a total width of 1 meter. The width of all other cells are 2 meter and the time step is chosen as 5 days to show a clear development over time of the plume growth and concentration in the graphs in figures 5 and 6.

Table 2 presents a selection of characteristic calculated ATP parameters used in the simulation. The difference between the two cases in terms of heterogeneity is reflected by the ratio of conductivities (k-ratio) and the wake ratio (β-ratio) and results in significant differences in the values of the specific shifted volume υj and the advective volume shift ω∆T,j. In the last column, the growth of the plume the drift per time step illustrates the strong asymmetry in the development of the plume for the case MADE while it is moderate for the case Cape Cod.

| Table 2 Selected characteristic calculated parameters in simulation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | kj [-] | βj [-] | υ(∆T) [-] | ω(∆T) [m] | Δs(∆T,dr) [m] |
| Cape Cod | Front | 0.61 | 0.52 | 0.0436 | 0.0190 | 0.2090 |
|  | Tail | 1.63 | 0.29 | 0.0557 | 0.0377 | -0.4806 |
| MADE | Front | 0.07 | 0.86 | 0.0557 | 0.0682 | 0.1680 |
|  | Tail | 14.09 | 0.03 | 0.0286 | 0.0716 | -2.4262 |

Figure 5 shows a 3D distribution of the plume development over time in the two cases, where the vertical axis gives the concentration the horizontal axis at the front of the graph represents the spread over the distance to the plume center and the third axis denotes the elapsed time. The 3D graphs show the development per time period of 25 days, showing the concentrations from each fifth calculation. The graphs represent the plume moving with average aquifer velocity of 0.5 meter per day, which means that the time step scale can be transformed to travel distance simply by multiplication by that velocity.

Figure 5. Concentration [%] change, colored per 10% range, over time [days] and spread [m] in longitudinal direction (x) while moving with plume center at x=0 (Lagrangian framework) for sites at Cape Cod (left) and MADE (right).

The drop over time in the concentration in the center of the plume in figure 5 is significantly faster for the case MADE than for the case Cape Cod, which reflects the large value of the advective volume shift of the front at MADE (table 2).

In figure 6, the concentration distribution is presented at logarithmic scale along y-axis to show the order of magnitude of concentrations, while the horizontal axis is similar to that in figure 5. The vertical dashed lines indicate the distance to the plume center of a cell for which multiple timesteps have been needed for a complete coverage by the plume drift.

Figure 6. Concentration [%] change at log scale (y) over time [days], colored per time step [25 days], and spread [m] in longitudinal direction (x) while moving with plume center at x=0  for sites at Cape Cod (left) and MADE (right). Dashed lines explained in text.

At low concentration, figure 6 shows the difference in the asymmetric development of the plume. The fast drop of the maximum concentration at MADE is indicated in Figure 8 of A&G 1992. The nearly symmetrical growth of the plume at Cape Cod is reflected in figure 2 Hess 2002.

Conclusions:

The method of advective transport phenomena (ATP) provides explicit and simple equations for the growth with traveled distance of the length, the so-alled spread, and thickness, the so-called wake, of a plume or cloud of water particles in groundwater. The plume dimensions are different for the front and the tail of a plume.

Continuity of flow means that water particles spread equally around the average travel distance. This means that mechanical dispersion consists of water particles traveling faster and slower than the aquifer average velocity.

The method of ATP describes mechanical dispersion by a new parameter, the so-called dispersive volume shift ω∆T,j, that describes the impact of the shift of water particles at sub-model scale over a time step ∆T in the front (j=f) or tail (j=t) of a plume as it occurs in addition to advection. Its value is determined by the stochastic parameters representing the aquifer heterogeneity and by the traveled distance in the time step. The dispersive volume shift comes at the position of the dispersivity parameter in the classic PDE (REF).

The concentration gradient equals the local concentration gradient, meaning between two calculation nodes, in the previous calculation step and is not considered as the driving force.

The applicability of the dispersive volume shift in a numerical is demonstrated by the results in figures 4 and 5.

The present work presents a parameter describing mechanical dispersion that differs from the dispersivity parameter describing the concentration distribution observed in field experiments. (De Lange, 2022) has shown that the results from ATP compare well to the observed values for 146 dispersivities reported from fourteen large scale field experiments.

Future developments

For a robust numerical model, the criteria for the optimum time step and cell-size for mass conservation have to be defined and tested.

The averaging of the concentration over the thickness of a cell versus the known affected thickness, the wake needs to be investigated and may lead to adaptation of the calculation scheme in specific cases.

The impact of the repetition of dispersion process may lead to significant impact on monitoring design as presented in De Lange 2024. This question needs to be answered for modeling practice and includes the issue of the known versus unknown traveled distance to the origin of the solvent.

Acknowledgement

The support and comments of the anonymous reviewer are greatly appreciated. The support in discussions and writing by Dr. Theo N. Olsthoorn, professor emeritus at Delft University of Technology, has led to considerable improvement of the manuscript. Many persons have supported the author along the road to the present work over the last 30 years, which is gratefully acknowledged.

Supporting information

As mentioned above

Author's Note

The author does not have any conflicts of interest or disclosures to report.

References

Adams, E.E.,, Gelhar L. W., 1992. Field Study of Dispersion in a Heterogeneous Aquifer 2 Spatial Moments Analysis. Water Resour Res 28, 3293–3307.

Adams, E.E., Gelhar, L.W., 1992. Field Study of Dispersion in a Heterogeneous Aquifer 2 Spatial Moments Analysis. Water Resour Res 28, 15.

Appelo, C.A.J., Postma, D., 1993. Geochemistry, groundwater and pollution. A.A. Balkema Rotterdam.

Batu, V., 2006. Applied flow and solute transport modeling in aquifers. Taylor & Francis, Boca Raton FL.

Bear, J., Verruijt, A., 1987. Modeling Groundwater Flow and Pollution. D. Reidel Publishing Company.

de Lange, W.J., 2022. Quantification of Advective Transport Phenomena to Better Understand Dispersion in the Field. Groundwater 60, 319–329. https://doi.org/10.1111/gwat.13151

de Lange, W.J., 2020. Advective Transport Phenomena to Better Understand Dispersion in Field and Modeling Practice. Groundwater 58, 46–55. https://doi.org/10.1111/gwat.12883

Eames, I., Bush, J.W.M., 1999. Longitudinal dispersion by bodies fixed in a potential flow. Proceedings of the Royal Society A: Mathematical, Physical and Engineering Sciences 455, 3665–3686. https://doi.org/10.1098/rspa.1999.0471

Feehley, C.E., Zheng, C., Molz, F.J., 2000. A dual-domain mass transfer approach for modeling solute transport in heterogeneous aquifers: Application to the Macrodispersion Experiment (MADE) site. Water Resour Res 36, 2501. https://doi.org/10.1029/2000WR900148

Fiori, A., Cvetkovic, V., Dagan, G., Attinger, S., Bellin, A., Dietrich, P., Zech, A., Teutsch, G., 2016. Debates—Stochastic subsurface hydrology from theory to practice: The relevance of stochastic subsurface hydrology to practical problems of contaminant transport and remediation. What is characterization and stochastic theory good for? Water Resour Res 52, 9228–9234. https://doi.org/10.1002/2015WR017525

Fiori, A., Dagan, G., Jankovic, I., Zarlenga, A., 2013. The plume spreading in the MADE transport experiment : Could it be predicted by stochastic models ? 49, 1–11. https://doi.org/10.1002/wrcr.20128

Garabedian, S.P., Leblanc, D.R., Gelhar, L.W., Celia, M.A., 1992. Natural gradient tracer in sand and gravel Cape Cod, Massachusetts, 2. Analysis of spatial moment of non-reactive tracer. . Water Resour. Res. 27.

Garabedian, S P, Leblanc, D.R., Gelhar, L.W., Celia, M.A., 1992. Natural gradient tracer in sand and gravel Cape Cod, Massachusetts, 2. Analysis of spatial moment of non-reactive tracer. Water Resour Res 27, 14.

Hess, K.M., Davis, J.A., Kent, D.B., Coston, J.A., 2002. Multispecies reactive tracer test in an aquifer with spatially variable chemical conditions, Cape Cod, Massachusetts: Dispersive transport of bromide and nickel. Water Resour Res 38, 36-1-36–17. https://doi.org/10.1029/2001wr000945

Konikow, L.F., 2011. The Secret to Successful Solute-Transport Modeling. Ground Water 49, 144–159. https://doi.org/10.1111/j.1745-6584.2010.00764.x

Molz, F., 2015. Advection, Dispersion, and Confusion. Groundwater n/a-n/a. https://doi.org/10.1111/gwat.12338

Neuman, S.P., 1990. Universal scaling of hydraulic conductivities and dispersivities in geological media. Water Resour Res 26(8), 10.

Strack, O.D.L., 1989. Groundwater Mechanics. Prentice Hall Inc.

Zheng, C., Wang, P.P., 1999. MT3DMS: A Modular Three-Dimensional Multispecies Transport Model for Simulation of Advection, Dispersion and Chemical Reactions of Contaminants in Groundwater Systems; Documentation and User’s Guide.

FIGURE Captions

Figure 1 Vertical section with equipotential lines (red) and path lines (green) for a specified travel time. A straight front of water particles (at left) is changed after passing through a zone with a conductivity five times higher (top) and lower (bottom) than that of the background of the aquifer. Blue lines indicate the initial and the final shape of the water particle front.

Figure 2 Vertical section. Scheme of spread and wake in figure 1, top and drift and reflux across the boundary of a domain A.

Figure 3. Sectional view of the shift of a water volume across the upstream (left) and downstream (right) boundaries of a cell (the black rectangle in the center) for a plume front (top) and tail (bottom)

Figure 5. Vertical section. Mass exchange between cells across upstream and downstream boundaries.

Figure 6. 3D plume shape development with traveled time [days], colored per 10% range in relative concentration for Cape Cod (left) and MADE (right).

Figure 7. Change over time [days] of spread [m] wand relative concentration [%] with logarithmic distribution along y axis for Cape Cod (left) and MADE (right).